Rover VVC

EU2

## 36-way ECU connector

24

25

36

1

12

13

Rear view of connector

| Pin number | Normal use | VVC Emerald ECU |
| --- | --- | --- |
| 1 | Injector driver 4 | ## Injector 2 |
| 2 | IACV | Channel 2 |
| 3 | IACV | Channel 1 |
| 4 | ## Main relay control | Main relay control |
| 5 | Ignition driver 2 | ### Coil 2 |
| 6 | Cooling fan relay control | Cooling fan relay |
| 7 | Oxygen sensor 1 signal input | 0-1v (narrow band) input |
| 8 | Throttle pot signal input | 0-5v input |
| 9 | Throttle pot supply / +5v out | TPS / MAP +5v supply |
| 10 | AuxIn10 | Oil temp sensor signal |
| 11 | Ignition sense input | ## +12v supply via ignition switch |
| 12 | Tacho output signal | 12v pulsed output |
| 13 | Immobilisor input | Immobilisor signal / Aux digital input |
| 14 | +8v out | Not used |
| 15 | Cam phase | Cam sensor signal |
| 16 | Air temperature signal input | ATS |
| 17 | AuxOut17 | VVC increase |
| 18 | Oxygen sensor signal earth | Lambda signal earth |
| 19 | Shift-light driver |  |
| 20 | ## Fuel pump relay driver | ## Fuel pump driver |
| 21 | AuxOut21 | VVC decrease |
| 22 | IACV | Channel 3 |
| 23 | Injector driver 2 | Injector 3 |
| 24 | Injector driver 1 | Injector 1 |
| 25 | Ignition driver 1 | Coil 1 |
| 26 | Injector driver 3 | Injector 4 |
| 27 | IACV | Channel 4 |
| 28 | +12v Ignition supply | +12v supply |
| 29 | ## ECU earth | System earth |
| 30 | Sensor earth (common earth) | Common sensor earth |
| 31 | Main trigger signal input | Crank sensor signal |
| 32 | Main trigger sensor earth | Crank / Cam sensor earth |
| 33 | Coolant temp signal input | CTS |
| 34 | AuxIn34 | MAP signal |
| 35 | AuxIn35 |  |
| 36 | AuxIn36 / AuxOut36 | AuxOut36 - Lambda heater relay control |

#### VVC loom RED connector

| Pin Number | Wire colour | Function | Emerald K3 plug |
| --- | --- | --- | --- |
| 1 | Red/yellow | Cam sensor + | 15 |
| 2 | Black/blue | Cam sensor - | 32 (join with wire from Red26) |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 | Black/white | Tacho output | 12 |
| 10 | Orange/green | IACV | 22 |
| 11 |  |  |  |
| 12 | Yellow/brown | Injector, cylinder 1 | 24 |
| 13 | Yellow/red | Injector, cylinder 2 | 1 |
| 14 | Yellow/white | Injector, cylinder 3 | 23 |
| 15 | Orange/blue | IACV | 27 |
| 16 |  |  |  |
| 17 | Yellow/orange | - | - |
| 18 |  |  |  |
| 19 |  |  |  |
| 20 |  |  |  |
| 21 |  |  |  |
| 22 |  |  |  |
| 23 | Black | Crank/Cam cable shield | 32 (join with wire from Red2) |
| 24 |  |  |  |
| 25 | Blue/purple | Crank sensor + | 31 |
| 26 | Blue/white | Crank sensor - | 32 (join with wire from Red2) |
| 27 | Grey | Lambda sensor signal | 7 |
| 28 | Green/grey | Lambda sensor earth | 18 |
| 29 |  |  |  |
| 30 |  |  |  |
| 31 |  | diagnostics | - |
| 32 | Brown/Black | Engine bay cooling fan relay | - |
| 33 | Pink/blue | IACV | 2 |
| 34 | Orange/grey | IACV | 3 |
| 35 | Yellow/black | Injector, cylinder 4 | 26 |
| 36 |  |  |  |

VVC loom BLACK connector

| Pin Number | Wire colour | Function | Emerald K3 plug |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 | Brown/black | VVC increase | 17 |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 | Red/green? | +5v sensor supply (MAP sensor) | 9 (join with wire from Black18) |
| 9 |  |  |  |
| 10 | Green/orange | Oil temp sensor signal | 10 |
| 11 | Grey/white | Engine bay temp signal | - |
| 12 | Yellow/green | Throttle pot signal | 8 |
| 13 | Pink/black | Sensor earth | 30 |
| 14 | Black/green | Air temp sensor signal | 16 |
| 15 | Pink/green | Coolant temp sensor signal | 33 |
| 16 | - | - | - |
| 17 | Yellow/red | Immobiliser input | 13 |
| 18 | Yellow/purple | +5v sensor supply (TPS) | 9 (join with wire from Black8) |
| 19 | - | - | - |
| 20 | Black/green | Oxygen sensor heater relay | 36 |
| 21 | Black | ECU earth | 29 |
| 22 | White/pink | Main relay control | 4 |
| 23 | Black/yellow | VVC decrease | 21 |
| 24 | - | - | - |
| 25 | White/black | Ignition coil 1 | 25 |
| 26 | White/black | Ignition coil 2 | 5 |
| 27 | Brown/pink | +12v from main relay | 28 |
| 28 | Blue/grey | Cooling fan relay | 6 |
| 29 | - | - | - |
| 30 | Black/purple | Fuel pump relay | 20 |
| 31 | Green/brown | - | - |
| 32 | - | - | - |
| 33 | White | Ignition sense | 11 |
| 34 |  |  |  |
| 35 |  |  |  |
| 36 | Yellow/purple | MAP sensor signal | 34 |